Map Color Identification [Devices]:

| Color | Device | Note |
| --- | --- | --- |
| Orange | Server-IDF-MDF |  |
| Blue | Standard Network Port | Access Layer |
| Yellow | Camera Location |  |
| Red | Door / Access Control Location | Door + Badge Reader |
| Purple | Remote Switch | POE Switches |
| Green | Wireless AP |  |

Patch Cable Identification [Devices]:

| Color | Device | Note |
| --- | --- | --- |
| Orange | Service Cable | Trunk ¦ Servers ¦ Uplinks |
| Blue | Standard Network User | Access Layer |
| Yellow | Camera Network | Access Layer |
| Red | Door / Access Control Network | Access or Trunk [Unifi = Trunk] |
| Orange | Remote Switch / Wireless AP | Trunk ¦ Servers ¦ Uplinks |

Cable Labeling Guide:

| If using Patch Panel: | If Direct to Switch | Expansion: |
| --- | --- | --- |
| SITE-LOCATION-PANEL.NUMBER | SITE-LOCATION-SWITCH.NUMBER | Adding letters to the number |

Examples:

| Market Hall, Cable from ground floor IT Room [Chad’s office] 2nd patch bay to first floor camera: | MHF-IDF.1.1 PNL01.01 |
| --- | --- |
| Market Hall, Cable from basement [MDF] to first floor camera, #12: | MHF-MDF.0.1 PNL01.12 |
| Market Hall, Cable from understairs switch to 3rd floor door #20 | MHF-IDF.1.2 SW02.20 |
| Market Hall, Cable from understairs switch to 3rd floor door #20, newly added | MHF-IDF.1.2 PNL02.20a |

Notes: